# Marshall Islands Maritime Investment Project

Oil Spill Contingency Plans

The proposed activities do not represent major spill risks; however, the possibility of spills still exists. Each port should have a spill management plan that should cover a variety of types of spills. If the port does not have a Port Spill Contingency Plan, then consideration should be given to the preparation of one as ports represent high spill risk operations and a major spill in the marine environment can have devastating impacts. An outline of a typical oil spill contingency plan for a port is provided in Annexure L.2.

In the absence of an overarching Port Spill Contingency Plan, the following Spill Management Procedure (Annexure 9.1) should be adopted as the minimum for the MIMIP.

L.1 - Spill Management Procedure

1	Distribution

This management Procedure is to be available to all workers, site supervisors and management staff working on the MIMIP.

2	Scope

This management procedure relates to the MIMIP only. It is to be read in conjunction with the ‘MIMIP ESMF and ESMP.

3	Purpose

To ensure construction activities meet the MIMIPs environmental management obligations to:

- Protect the health and safety of all employees and ensuring that activities are not harmful to the environment and the greater community.

- Comply with ‘general environmental duty’ and not cause ‘Environmental Harm’ or ‘Environmental Nuisance’ in accordance with the Environmental Protection Act.

4	Performance Criteria

- All spills are reported to BLC Environment Staff within one hour of occurrence;

- No spills enter the local stormwater system or marine environment; and

- No occurrence of Class 1 or 2 incidents (refer below) as a result of a spill.

| Category | Description |
| --- | --- |
| Class 1: | Potential breaches of legislation or failure of process that result in actual off-site environmental harm, or residual on-site environmental harm; or Works undertaken outside approved areas, without required approval or without environmental assessment; or Any Material Harm pollutions incident as defined by the law. |
| Class 2 | Failures of process or events that do not result in off-site environmental harm, or residual on-site environmental harm. These incidents may result in temporary on-site environmental harm that can be rectified to pre-existing conditions |
| Reportable Event | An event or unexpected find that occurs outside the scope of reasonable environmental controls and mitigation measures |

5	Management Measures

5.1	General Procedure

Source of leak or spill unable to be controlled by Site Staff.

- Notice spill

- Assess risk

Assess your safety and that of others, e.g. overwhelming odour, flammable liquids, etc. Take appropriate precautions (e.g. evacuate, utilise personal protective equipment). Seek advice from Port Manager. Locate any drains or waterways near the spill site that the contaminant is likely to enter.

- Inform

Immediately notify the Site Supervisor. The Site Supervisor is to immediately inform Safeguards Specialist.

- Control

Find the source of leak or spill. Check if it is still occurring. If safe, stop the leak/spill, e.g. by closing valve, plugging leak. If not safe, do not attempt to stop the leak until further help is sort.

Contact emergency services. Follow their instructions.

- Contain

Do not spread or dilute spills with degreasers, detergents or water. Coordinate all containment actions through Safeguards Specialist. High priority is to be given to those spills which have the potential to enter the marine environment. See Section 5.2 for specific methodologies.

Leak or spill unable to be contained by Site Staff.

- Clean-up

Clean spill site with absorbent material e.g. peat, pads and booms found in spill kits. See Section 5.2 for specific methodologies. Dispose used absorbents into contaminant disposal bags within spill kits. Wash down non-disposable equipment in a bunded vehicle wash-down area. Supervisor to organise replacement spill kit.

- Rectification

Safeguards Specialist to inspect spill site and ensure clean-up is satisfactory. Site Supervisor to monitor source or activity that caused leak and improve equipment or process to prevent future occurrence of leak. If required, Plant Supervisor is to be notified of faulty equipment and maintenance undertaken to fix the source. Safeguards Specialist to review spill details and complete an Incident Report in conjunction with those involved. Report to be reviewed and signed by the Project Manager.

- Notification

Depending on the severity of the spill, the EPA or other regulating agencies are to be notified of the incident by Safeguards Specialist. Any directives given by the EPA or other regulating agency are to be recorded and followed.

Safety concerns can be managed by Site Staff.

Safety concerns cannot be managed by Site Staff.

Spill site unable to be cleaned-up by Site Staff.

HSE staff to liaise with EPA & coordinate clean-up with external contractors.

5.2	Specific Methodologies

|  | Spills on Water | Spills on Land |
| --- | --- | --- |
| Hydrocarbons | Contain Obtain booms from hydrocarbon spill kits. Position booms in waterbody at a 45º angle to direction of flow. Take care to place booms in slow-moving water at a point where the oil/fuel has settled to the surface. Ensure booms are overlapped when tied together. Position a back-up boom 1-2 m downstream of the first boom. Clean-up Apply peat and/or pads from hydrocarbon spill kits across surface of contained spill. Wait for oil/fuel to be adsorbed/absorbed (a few minutes). Remove peat and/or pads by skimming water surface. Repeat as necessary until all oil/fuel is removed. Remove booms. Place all contaminated materials in plastic contaminant bags, tie-off bags, clearly label & arrange for disposal. | Contain Contain spill to immediate area by placing booms from hydrocarbon spill kits around the spill site. Place absorbent pads behind booms in places where seepage is likely to occur. If no booms or pads are available, use earth mounds to contain spill. Clean-up If on gravel or soil, apply peat to spill area, wait for adsorption to take place (a few minutes) and then excavate all contaminated material. Replace with clean fill. If on a hardstand area, apply peat or pads, wait for adsorption/absorption to take place (a few minutes) and then brush up material into plastic contaminant bags. Place all contaminated materials in plastic contaminant bags, tie-off bags, clearly label & arrange for disposal. |
| Chemical | Methodology dependent on chemical type and amount spilled. Refer to MSDS sheet and consult the Port Manager | As per methodology for a hydrocarbon spill on land, except use chemical spill kits (materials within these kits are hydrophilic as opposed to hydrophobic). Refer to MSDS sheet and consult HSE staff |

5.3	Spill Kits

Spill kits are to be available at the port site office and in the supervisors’, foremen, environment and safety staff vehicles. The spill kits should be contained within yellow bags or similar labelled, “Spill Kit”.

Each kit should contain:

2 x 2.4m x 75mm mini booms;

12 x absorbent pads;

2 x absorbent pillows;

1 x Global Peat 6-liter bags;

2 x disposal bags; and

1 x instruction sheet laminated.

5.4	Staff Training

Safeguard Specialist is to ensure that all site staff know where to access a spill kit and what to do in the event of a spill. This is to be achieved through:

Signage for locations of spill kits;

Specific spill training workshops;

Site inductions;

Work Activity Briefings (including JSEAs); and

Toolbox talks.

6	Roles and Responsibilities

All Staff

All staff have an immediate duty to report actual or potential/near-miss spills to their respective supervisor.

Site Supervisors, Foremen and Leading Hands

To report all spills to Safeguards Specialist within one hour of occurrence.

To assist Safeguards Specialist with any inquiries when undertaking incident investigations, including suggestions to improve equipment or process to prevent future occurrence of leak.

To have a spill kit readily available at all times when conducting works and know how to use the kit effectively.

To ensure used absorbent material is taken for disposal by a licensed waste contractor.

To ensure used spill kits are replaced as soon as practical after use.

Health and Safety Staff

To ensure an adequately stocked spill kit is available on their vehicle at all times.

To advise of any health and safety considerations when treating or cleaning up a spill.

Safeguards Specialist

To inspect the availability of spill kits around site during weekly site inspections.

To conduct or organize staff spill training.

In the event of a spill:

To inspect the spill site and ensure the cleanup is satisfactory;

To review spill details and complete an Incident Report; and

To liaise with the EPA and/or other regulating agencies.

7	Monitoring and Compliance

The occurrence of spills and conformance to this procedure will be monitored by BLC Environment Staff through weekly site inspections, water quality testing (BLC-PR-00104) and incident reporting (PRG-FM-00010-02) and summarized in the Monthly Environmental Report. Corrective actions may include further training (e.g. toolbox talks), the issue of a Site Instruction or disciplinary action, depending on the severity of the non-conformance.

Any directives given by the EPA or other regulating agency are to be recorded and followed in the event of a spill.

8	Reference Documentation

MIMIP ESMF and ESMP

9	Key Contacts

List key contacts as appropriate e.g.:

Port General Manager –

Seaport Manager –

Safeguards Specialist –

HSE staff –

EPA -

L.2: Sample Table of Contents for Port Oil Spill Contingency Plan

- Introduction

  - Authority for the plan’s aims and objectives of the plan

  - Geographical scope of the plan

  - Integration with other plans

- Notification and Evaluation

  - Reporting and notification procedures

  - Initiation of response

  - Spill response teams

  - Advanced operations center

  - Estimate of risk

  - Tiered response (small, medium and large spills)

  - Movement and fate of oil on the water

  - Incident scenarios

- Oil Spill Response

  - Response options

  - Monitor the oil slick

  - Contain and recover the oil from the sea surface

  - Apply oil spill dispersants

  - Physical breakup of oil using surface craft

  - In-situ burning

  - Physical protection of the area and its resources

  - Cleanup of oiled foreshores

  - Bioremediation

  - Oiled waste management

  - Environmental sensitivity

  - Coastal bird populations

  - Oiled wildlife operations

  - On-scene liaison

  - Recording and cost recovery

  - Termination of clean up

  - Clean down and restoration of equipment

  - Post-spill damage assessment and monitoring

  - Post-spill restoration and rehabilitation

- Supporting Information

  - Relevant legislation

  - Roles of authorities

  - Description of port and adjacent area

  - Seasonal conditions affecting the response

  - Oil spill trajectory modelling

  - Equipment maintenance and storage

  - Health and safety considerations

  - Training

  - Maintenance of the plan