Marshall Islands Marine Improvement Project (MIMIP)

Design and Supervision of Safe, Efficient and Climate Resilient Maritime Infrastructure in RMI

ESMP and Technical Documents

World Bank Comments

Detailed Design Report, the Basis of Design Report, the Condition Report, the Pavement Design Report, the Early Works drawings

1. General Comments

Detailed Design Report (+16 Appendices)

- It is unfortunate that the report is not complete - specifically missing the cost estimate, the Uliga control tower detailed design, the final design drawings, specifications etc.

- The report has been prepared in an unusual, but very practical, way. For each activity at each dock, the detailed design update has been added to the preliminary design text. This facilitates the review, as a significant portion of the activities at each dock remain unchanged from the preliminary design.

- We feel that the report is technically sound, when read in conjunction with the Basis of Design report and other reports in the appendices. It clearly sets out how the design decisions were reached and details of the proposed improvements, as well as identifying risks.

Detailed drawings

- Similarly, these have generally been well prepared and provide sufficient detail for construction to proceed.

- The drawings of Uliga control tower only provide architectural floor plans and an artist's impression of the building. They are not construction drawings.

Timeline

- This is now academic; however, the 3-year program shown for construction of the works as designed seems reasonable.

Early works

- The drawings are included could not find BoQs, specs or cost estimates.

2. Specific comments

| WB comments | RMIPA comments | Consultant comments |
| --- | --- | --- |
| The recommended concrete design strength in pavement and structures in the Basis of Design Report and on the drawings is C50 - 50Mpa. However, the Detailed Design Report (section 10 Table 10-1) recommends C40 for durability reasons. RHDHV needs to check that all its outputs are consistent on this and other chosen design parameters. |  |  |
| Standby generator foundation slabs are included at Delap and Uliga (and Ebeye?) but unless the size of the generators are known in advance, it would be premature and possibly abortive to specify a foundation size. |  |  |

Environmental and Social Management Plan (ESMP)

1. General Comments

- Unfortunately, the ESMP appears to be hastily compiled, hence the number of comments below in the document. We would suggest:

  - That the report includes an executive summary, focussing on the key risks and impacts, and recommended mitigation measures.

  - Includes stronger references/interlinkages to the ESMF and the Marine Ecological Assessment (Appendix A1)

  - In the impact assessment sections, discuss the potential impacts for each site, associated with the activities proposed for that site. At present, the reader is required to refer back to the tables in section 1, as the activities are also not listed/summarised in Table 6-4.

- The code of conduct does not appear to be included in the ESMP or the bidding documents, with the bidding documents referring back to the ESMP. We would suggest that the ESMP includes the code of conduct and makes clear that these would need to form part of the bidding documents.

2. Specific comments

| Section / Reference | WB comments | RMIPA comments | Consultant comments |
| --- | --- | --- | --- |
| Main Document |  |  |  |
| 1.3 | p8 – p7 of the ToR r5 includes a long point about consultation. |  |  |
| 1.4 | p9 – Why doesn't the ESMP cover C1? Please include explanation. |  |  |
| Figure 1-1 | please show the correct location of Delap Dock (same for the other figures). |  |  |
| Table 1-1 | it would be helpful to start with a list of the activities proposed, specific to Delap; and so on for the other sites. Refer to the section/tables in the main report. |  |  |
| 2.1 | p18 – please include a reason/justification for the choice of these sites only, for each specialist study. |  |  |
| 2.2 | p18 – 3rd paragraph – Please direct the reader to the survey location figures in A1. |  |  |
| 2.3 | p 19 2nd paragraph – Majuro, or Ebeye? |  |  |
| 2.3 | p20 – was there no water quality or sediment testing at Ebeye? Need to explain why conducted at Majuro but not here. Also for other sites. What are the proposed disposal methods for hazardous sediments, and recommended mitigation measures for mobilisation of the sediments that have potential to release contaminants into the water column during the works? |  |  |
| 2.5 | p21 last paragraph – please include a location map showing the protected area (also for Wotje and Arno). Also refer to the figures in Appendix A, however note that due to the size and resolution, the Protected Areas appear to cover the entire atoll, so clarification at the project sites is required. |  |  |
| 2.6 | the paragraphs are confusing, switching between topics, zones and areas. Please review and clarify, removing apparent contradictions. Reference to the study in Appendix A, including page numbers for the relevant data, would be beneficial. The marine survey in Appendix A is a comprehensive report and should be concisely summarised in this section (for example a summary of the key findings, potential impacts and recommended mitigation measures for each site). |  |  |
| 2.7 | p24 – the statement of no impact due to distance for protected areas at all sites requires further justification and clarification – eg which direction is the dominant current flow (thereby identifying that the current will take the potential sediment load away from sensitive areas), and where are the natural/existing accretion or reduction areas? |  |  |
| 5.2.1.2 | p30 – why is “Construction Use” located under 5.2.1.2 Food Supplies? Suggest moving to a more relevant section. |  |  |
| 5.2.1.3 | p30 – please provide more information – how will the wastewater be collected, and how managed at the local landfill? |  |  |
| 5.3 | p30-31 - Please refer to ESMF for waste management procedures (WM2; Appendix P) and also note the requirements for other islands. “WM2.3: All solid waste is transported off-island for recycling or disposal at a licensed and engineered landfill, with appropriate permits and approvals under international treaties such as Waigani.” Appendix P: “No RMI landfills will be used for any waste. All waste is to be recycled or disposed offshore at a permitted facility. No dumping of any waste in RMI”. |  |  |
| 5.3 | p31 2nd paragraph – please see comment above, and ensure relevance to RMI landfills. For example, the Majuro landfill is not engineered, and is not consistent with international industry good practice, and is already significantly overburdened. |  |  |
| 5.3 | p31 – please review references to Appendices for accuracy. |  |  |
| 5.3 | p31 paragraph 6 – please ensure relevance to RMI EPA requirements, and/or state that NSW EPA is best practice and therefore required to be followed, and/or is being used as the benchmark. |  |  |
| 5.3 | p32 – please confirm that there is space for stockpiles at the ports that is not operational space. What type of “future use” is envisaged? How long is it anticipated that the “significant amount” will be stockpiled for? |  |  |
| 5.4 | p32 – on these atolls there is generally only one haulage route. |  |  |
|  | will there be a mobile concrete batch plant at each site? |  |  |
| 5.5 | p33 – what is meant by: “Business Continuity Management – interruptions to the delivery of Royal HaskoningDHV services and activities may require a Business Continuity Response Team to assist in returning to business as usual.” |  |  |
| 5.8 | 8 p33 – why was water quality sampling only undertaken at Delap and Arno? Please include explanation re why not conducted at all sites. |  |  |
| 5.8 | p34 – please provide examples of the “effective mitigation measures during construction”. |  |  |
| 5.9 | Flora and fauna impacts section is lacking detail, and seems disjunct from the descriptions of benthic environment in earlier sections, and the marine survey documented in Appendix A. |  |  |
| 5.9 | p34 – were any surveys undertaken? If so, reference them. |  |  |
| 5.9 | p34 – are there nesting areas in/near any of the project sites? Elaborate on the statement in para 2 re unsuitability. Please provide references; such statements cannot simply be made; please reference the marine/benthic environment descriptions and habitat assessments for each site. |  |  |
| 5.9 | p34 Jaluit dock “supports a very low hard coral…” – very low what? Depth, quality, biodiversity? |  |  |
| 5.10 | p35 – describes Ebeye – what about the other sites? Please attach the UXO study/studies to the ESMP. Why was Wotje the only site chosen for physical survey (please provide justification)? |  |  |
| 6 | p36 Table 6-1 and 6-2 – please correct the headings for the tables. |  |  |
| Table 6-4 Mitigation Plan | Please include a column for monitoring and reporting – type, frequency, and responsibility. |  |  |
|  | The items listed in the first column aren’t “Project Activities” – please review headings and contents. |  |  |
|  | The mitigation matrix needs to include: - GBV/SEAH mitigation. - Social risks related to new leases, including proper community consultation. |  |  |
|  | Please: - cross-check all items against the requirements in the ESMF, i.e. Tables 3 to 14. - review the statements relating to portable buildings and desalination plant for Delap. - include UXO as a separate risk, as it has potential impacts on workers and communities; include a Chance Finds Procedure in the ESMP to provide instruction on what to do if UXO is encountered. - provide an outline Waste Management Plan in the ESMP - add “lagoon” to “No sand or aggregates will be sourced from…”; and include reef rock. Or consider a broader statement that no local materials can be used. - remove the reference to an “Erosion Sediment Control Plan (ESMP)”. |  |  |
|  | Flora and fauna – the contents of this row (apart from the lighting aspect) relate predominantly to the AtoN work. Please incorporate the recommendations from the marine ecological assessment, and ensure all aspects of each sub-project and location are addressed. |  |  |
| 7 | The GRM should include GBV channels and contact points. They should refer to the project's most recent GRM document. |  |  |
| Appendix A1 | There is a lot of useful information here. Please ensure the key points are referred to and referenced in the main document. |  |  |
|  | Please conduct a spelling, typo and grammar check throughout. |  |  |
| 1.0 | p6 – please include a statement justifying why only undertaken for Arno, Jaluit and Wotje, and not the other sites. |  |  |
|  | 2nd last dot point – will the works result in any disturbance? |  |  |
| 1.0 | p8 Potential Impacts – consider use of “not significant” or “negligible”, rather than “insignificant”, here and elsewhere. |  |  |
| 1.0 | top of p9 – this appears to contradict the main report (p23), and p32 of this appendix, which states that there has been degradation of coral reefs as a result of dredging – is this project going to allow this to happen again, as that would not be “insignificant”? |  |  |
| 2.0 | p10 – (paragraph above Figure 1) please avoid an unwieldy single sentence becoming a paragraph (here and elsewhere). |  |  |
| 2.0 | p11 – what is the relevance of lagoon-based aggregates to this project? |  |  |
| 2.2 | p11 – typo – “projects development…” – please amend to “project’s” here and elsewhere. |  |  |
|  | As above – please include reason for only assessing Arno, Jaluit and Wotje. |  |  |
|  | RHDHV to confirm the summary of activities is accurate and current. |  |  |
| 2.3 | orientation of the plates being split over pages is confusing – consider each figure being a plate, or combining in some way. |  |  |
| 2.4 | 16 last paragraph – please clarify – is the Arno dock inshore, or seaward side? |  |  |
| 2.4 | p17 2nd paragraph is confusing and doesn’t follow from the previous para. |  |  |
|  | It would be helpful to include a table of the species recognised for conservation in RMI. |  |  |
|  | “The 18 threatened species… The other 43 species…” – which 18, 43? |  |  |
|  | The next paragraph refers to 19 endangered species. Please review this entire section for accuracy, consistency and flow. |  |  |
| 2.4.2 | p19 – This section on Arno doesn’t describe the habitat disruption caused by the dredging, as noted in the main body of the ESMP – reason for this? |  |  |
|  | The statement in the last paragraph regarding no impact from the scope of works is seemingly based on distance only. Please provide further clarification in relation to the hydrodynamics of the coastline at the study site, eg re dominant current direction, existing accretion patterns, etc. that will (or won’t) prevent siltation of the protected areas. This comment also applies to the other sites. It is noted that s2.6 refers to data collection “…including water movements/currents.” |  |  |
| 2.4.2 | p20 Plate 4 – please make clearer – also show how the study site is outside the PAN: at the scale shown it looks like the whole atoll is included in a category. This comment also applies to the other plates. |  |  |
| 2.5 | p22 – states SCUBA was used for surveys but then only describes snorkelling location and frequency. 2.6 also mentions SCUBA. |  |  |
| 2.6 | p23 Table 3 missing month for Wotje. |  |  |
|  | p23 last para – please clarify re free dive vs scuba dive. |  |  |
| 3.1 | top of p27 – please review this sentence. |  |  |
|  | Plate 9 – (a) and (b) are not annotated on the plate. |  |  |
|  | See comment above re consistency: p32 states “These elevated levels of suspended sediment has resulting in elevated mortality levels of hard corals over an extended area and period of time (Plate 13). This mortality is unacceptable and urgent sediment mitigate and management measures need to be developed and implemented at the Arno dock and surrounding area to prevent further negative impacts on the marine environment and its resources.” p34&36 recommends cleanup to remove rubbish. |  |  |
|  | Last dot point p36 – please review wording. |  |  |
|  | Top of p39 – please review wording. |  |  |
|  | Wotje Dock p58 – check wording in last dot point for impacts: ‘… supports a low hard coral and seagrass…” |  |  |
| Appendix A2 & A3 | As noted above, please include justification in main doc for why only these sites. |  |  |